# 1. Product Overview

## 1.1 Product Features

The TS-E-A series dual-finger tactile sensor for robotic grippers is an intelligent bionic skin solution independently developed by Tashan Technology for robotic grasping perception.

It enables:

- Three-dimensional force sensing (normal and tangential forces)

- Proximity detection

- Pressure sensing

- Material recognition

The sensor mimics the physical characteristics and tactile capabilities of human finger skin and is highly compatible with a wide range of robotic grippers.

It features strong versatility, high sensing accuracy, and enables stable and precise grasping performance.

## 1.2 Intended Use and Application Scope

The sensor is primarily used for:

- Non-contact object position detection

- Object material identification

- Normal force and tangential force measurement

- Adaptive grasping in conjunction with robotic arms

It has broad application prospects in:

- Scientific research and education

- Biomedicine

- Space exploration

- Hazardous environment operations

- Industrial manufacturing

- Service robotics

## 1.3 Operating Environment

- Operating Temperature: -10°C to 45°C

- Storage Temperature: -20°C to 70°C

- Humidity: Non-condensing

## 1.4 Product Appearance

# 2. Structural Characteristics and Working Principle

## 2.1 Overall Structure and Operating Environment

### Structure

The tactile sensor consists of:

- Flexible silicone skin surface

- Rigid Printed Circuit Board (PCB)

- Insulated plastic base housing

### Operating Environment

The sensor utilizes non-contact capacitive detection. Since capacitance is sensitive to environmental factors such as temperature and humidity, it must be operated in a relatively stable environment within specified conditions.

Caution:
 Prolonged overload pressure on the flexible silicone may lead to incomplete rebound. Long-term operation beyond the rated range must be avoided.

## 2.2 Functional Units and Interconnections

The system consists of:

- Sensor Units (Dual)

- Main Controller

The main controller supports up to five sensor units simultaneously. A host computer must be provided by the user.

### Sensor Unit

The sensor unit transmits:

- Raw capacitance values (caused by silicone deformation)

- Calibrated force values

via the I²C protocol to the host system.

### Host System

Using dedicated host software, users can:

- Monitor sensor data

- Update firmware

- Configure parameters

- Perform debugging

# 3. Technical Specifications

## 3.1 Main Functions

### Sensor Unit (Dual Finger)

- Non-contact proximity detection

- Algorithm-based material recognition

- Contact-based adaptive grasping

### Main Controller

- Processes and transmits finger data to the host computer

## 3.2 Key Parameters

### Normal Force

- Range: 50 N

- Minimum Detectable Force: 0.1 N

- Resolution: 0.1 N

- Accuracy: 5% FS

### Tangential Force

- Range: 20 N

- Minimum Detectable Force: 0.25 N

- Resolution: 0.25 N

- Directional Resolution: 45°

- Directional Range: 0°–359°

### Sampling

- Capacitance Sampling: 24-bit (dimensionless)

- Sampling Frequency: ≥ 50 Hz

### Electrical Characteristics

- Rated Voltage: 3.3V / 5V

- Maximum Static Operating Current: 20 mA

### Installation

- Mounting: Bolt-fixed to robotic gripper connection plate

- Operating Position: Sensor surface

### Connection

- Connector: SMD 4-Pin, 0.8 mm pitch

- Cable: 4-core, 0.0316 mm²

### Mechanical Dimensions

- 46.7 mm × 20.2 mm × 12.5 mm

### Environmental Conditions

- Operating Temperature: -10°C to 45°C

- Storage Temperature: -20°C to 70°C

- Humidity: Non-condensing

## 3.3 Dimensions and Cable Length

- Cable Length: 200 mm

# 4. Installation and Commissioning

## 4.1 Hardware Installation

### Finger Installation

Securely mount the sensor fingers onto the robotic gripper (e.g., DH Robotics gripper or equivalent).

### Interface Definition

  - The internal module uses a 0.8 mm pitch IDC socket.

  - The I²C adapter board utilizes the PCA9548APWR for multi-channel I²C switching to enhance communication reliability.

  - USB-to-I²C conversion is handled by the CH341 chip.

## 4.2 Software Installation

### CH341 Driver Installation

- Run CH341PAR.EXE

- Click Install

- Confirm installation and close the installer

## 4.3 Debugging Procedure

### 4.3.1 Preparation

- Install USB-to-I²C driver

- Connect sensor to adapter board

- Connect adapter board to PC via USB-C

## 4.3.2 Adapter Board Description

The adapter board consists of:

- CH341 (USB-to-I²C interface)

- PCA9548 (I²C multi-channel switch)

- PCA9548 I²C Address: 0x70

| PCB Interface (Device No.) | PCA9548 Channel | Description |
| --- | --- | --- |
| Device 1 | Channel 4 | Standard Sensor Connection |
| Device 2 | Channel 5 | Standard Sensor Connection |
| Device 3 | Channel 6 | Standard Sensor Connection |
| Device 4 | Channel 3 | Standard Sensor Connection |
| Device 5 | Channel 2 | Standard Sensor Connection |
| Debug Port | Channel 0 | External logic analyzer connection for I²C protocol monitoring |

## 4.3.3 Demo Host Software Operation

### 1. Device Connection

- Open SensorShow.exe

- Connect adapter board to PC

- Click Connect

- System automatically scans and reads connected sensors

### 2. Finger Information Interface

Displays:

- Firmware version

- Device type

- I²C address

- Sampling rate

- 3D force results

Visualization:

- Normal Force → color intensity

- Tangential Force → arrow length

- Tangential Force Angle → arrow direction (0°–359°)

- Proximity → bar chart variation

### 3. Data Interface

Displays real-time sampling data of all channels.

- Output modes:

  - Raw values

  - Filtered values

  - Baseline values

  - Difference values

- Channel reset allows recalibration

### 4. Settings Interface

Provides:

- Device reset

- Output type selection

- I²C address configuration (0x01–0x6F)

- Channel reset

- Channel value adjustment (~3,000,000 baseline)

### 5. Data Saving

- Format: CSV

- Directory: /data folder

- Naming:
 YYYY-MM-DD-HHMMSSms__sensorID.csv

### 6. TCP Data Transmission

The software acts as a TCP client.

Format:

data1,data2,...,data14\r\n

### 7. Firmware Update

- Connect device

- Select device ID

- Select .bin file

- Enable Software Reset

- Click Start

# 5. Operation and Use

## 5.1 Pre-Use Inspection

- Verify environmental conditions

- Ensure applied force is within range

- Inspect silicone surface

- Check mounting stability

- Confirm software operation

## 5.2 Safety Precautions

- Avoid overload

- Prevent damage from sharp objects

- Do not disassemble or modify

- Stop operation immediately if abnormal behavior occurs

## 5.3 Data Acquisition and Protocol

### I²C Communication

- Standard Mode: 100 kHz

- Fast Mode: 400 kHz

- Default address: 0x6F

- Configurable range: 0x01–0x6F

- Broadcast address: 0x00

### Checksum Calculation

Example:

Command:
 aa 55 03 71 00 00 00 00 00

Checksum:
 73 01

(C implementation consistent with other Tashan sensors)

# 6. Maintenance and Care

## 6.1 Storage Requirements

Store the sensor in an anti-static bag in a cool and dry environment.

## 6.2 Routine Maintenance

- Operated by trained personnel

- Follow product manual and procedures

- Avoid underwater or point-load usage

- Regularly tighten mechanical components

- Stop operation if abnormalities occur

# 7. Additional Information

## 7.1 Warranty

- 1-year replacement

- 2-year repair (non-human damage)